# XALTRAX

# 6-DOF Robotic Arm - Full Design & Implementation Guide

# Overview

This document provides a complete guide to building a 6 Degrees of Freedom (DOF) robotic arm. It includes mechanical design steps, electronics circuit design, control systems, component list, simulation tools, and use cases.

# 1. Components Required

Mechanical Components:
- Base platform (cylindrical or square)
- Link 1 (shoulder)
- Link 2 (elbow)
- Link 3 (forearm)
- Link 4 (wrist pitch)
- Link 5 (wrist yaw)
- Link 6 (end-effector/gripper mount)

Electronics:
- 6x Servo Motors (MG996R, DS3218, or Dynamixel)
- 1x Arduino Uno or Mega
- 1x Ultrasonic Sensor (HC-SR04)
- Breadboard
- Jumper wires
- External 5V-7.4V power supply (capable of 2A+ current)
- USB cable (for Arduino programming)
- Switches (optional)
- Voltage regulator (if needed)

Tools & Materials:
- Autodesk Fusion 360 (for CAD modeling)
- Soldering iron
- Screwdriver
- 3D printer (optional)
- Screws, bolts, spacers

# 2. Use Cases

- Education: Robotics training, kinematics demonstrations
- Industrial: Pick and place operations, light assembly tasks
- Research: Motion planning, AI integration, sensor feedback testing
- Home Automation: Small object manipulation, telepresence

# 3. Mechanical Design Steps (Fusion 360)

Model Each Component:
- Use "New Component" for each segment
- Sketch each part (rectangle or circle)
- Use Extrude and Fillet for structure
- Apply As-Built Joints (Revolute) to allow motion

Recommended Dimensions:
- Base: 10x10x2 cm
- Link 1: 3x3x15 cm
- Link 2: 3x3x25 cm
- Link 3: 3x3x20 cm
- Wrist: 3x3x5 cm (for each link)
- End-effector: 4 cm diameter x 5 cm

# 4. Circuit Design (Arduino + Servos + Sensor)

Connections:
- Servos: Connect signal pins to Arduino D2–D7
- Ultrasonic Sensor: Trigger to D8, Echo to D9
- Power: External 5V supply to servo VCC, GND common with Arduino
- Arduino powered via USB or 5V adapter

# 5. Software (Arduino Sketch)

Use Servo.h library
Control each servo by mapping angles (0–180)
Use ultrasonic sensor for object detection or collision avoidance

Sample Code Outline:
#include <Servo.h>
Servo s1, s2, s3, s4, s5, s6;
void setup() {
  s1.attach(2); s2.attach(3); s3.attach(4);
  s4.attach(5); s5.attach(6); s6.attach(7);
}
void loop() {
  s1.write(90); s2.write(45); s3.write(135);
  s4.write(90); s5.write(90); s6.write(90);
  delay(1000);
}

# 6. Kinematics Model (Python - SymPy)

- Use DH parameters
- Define 6 matrices
- Multiply to get T0_6 (end-effector transformation)

# 7. Simulation Tools

- Fusion 360 (animation)
- Python (matplotlib + SymPy)
- ROS + RViz for full robot simulation

# 8. Output/Applications

- Move object from point A to B
- Scan surface using sensor
- Follow programmed path
- Respond to distance or motion triggers

# 9. Future Improvements

- Add camera module
- Gripper with force sensors
- Feedback with encoders
- AI-based motion control

Additionally, for the simulation in Python, refer to the file on GitHub with the extension.py and pre-download these libraries

import numpy as np

import matplotlib.pyplot as plt

from matplotlib.animation import FuncAnimation

to download libraries

pip install libraryName